Page 1

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

Program Address Decoder for MultiSync II, Location 200M

J. Margolin     10 January 1992

Equations for Module _MS2_1

Device MS2_1

- Reduced Equations:

!nROM7 = (A18 & A19 & A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# A17 & A18 & A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# A18 & A19 & A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM6 = (!A18 & A19 & A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# !A17 & A18 & A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# !A18 & A19 & A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM5 = (A18 & !A19 & A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# A17 & !A18 & A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# A18 & !A19 & A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM4 = (!A18 & !A19 & A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# !A17 & !A18 & A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# !A18 & !A19 & A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM3 = (A18 & A19 & !A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# A17 & A18 & !A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# A18 & A19 & !A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM2 = (!A18 & A19 & !A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# !A17 & A18 & !A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# !A18 & A19 & !A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM1 = (A18 & !A19 & !A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# A17 & !A18 & !A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# A18 & !A19 & !A20 & !A21 & !A22 & !A23 & P0 & !nAS);

!nROM0 = (!A18 & !A19 & !A20 & !A21 & !A22 & !A23 & !P1 & !nAS

# !A17 & !A18 & !A19 & !A21 & !A22 & !A23 & !P0 & P1 & !nAS

# !A18 & !A19 & !A20 & !A21 & !A22 & !A23 & P0 & !nAS);

Page 2

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

Program Address Decoder for MultiSync II, Location 200M

J. Margolin     10 January 1992

Chip diagram for Module _MS2_1

Device MS2_1

P16V8S

----------\       /----------

|          \     /          |

|           -----           |

P0 |  1                    20  |

|                           |

P1 |  2                    19  | nROM0

|                           |

A17 |  3                    18  | nROM1

|                           |

A18 |  4                    17  | nROM2

|                           |

A19 |  5                    16  | nROM3

|                           |

A20 |  6                    15  | nROM4

|                           |

A21 |  7                    14  | nROM5

|                           |

A22 |  8                    13  | nROM6

|                           |

A23 |  9                    12  | nROM7

|                           |

| 10                    11  | nAS

|                           |

|                           |

-----------------------------

end of module _MS2_1

Page 1

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

Program Device Selector for MultiSync II, Location 185U

J. Margolin     13 January 1992

Equations for Module _MS2_2

Device MS2_2

- Reduced Equations:

!HM31A = (!A15 & !P0A & !P1A # P0A & !P1A & !nRWS & !nUDS);

!LM31A = (!A15 & !P0A & !P1A # P0A & !P1A & !nLDS & !nRWS);

!HM29A = (!P0A & !P1A & !nRWS & !nUDS # !A15 & P0A # !A15 & P1A);

!LM29A = (!A15 & P0A # !A15 & P1A # !P0A & !P1A & !nLDS & !nRWS);

!HM31B = (!A15 & !P0B & !P1B # P0B & !P1B & !nRWS & !nUDS);

!LM31B = (!A15 & !P0B & !P1B # P0B & !P1B & !nLDS & !nRWS);

!HM29B = (!P0B & !P1B & !nRWS & !nUDS # !A15 & P0B # !A15 & P1B);

!LM29B = (!A15 & P0B # !A15 & P1B # !P0B & !P1B & !nLDS & !nRWS);

Page 2

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

Program Device Selector for MultiSync II, Location 185U

J. Margolin     13 January 1992

Chip diagram for Module _MS2_2

Device MS2_2

P16V8S

----------\       /----------

|          \     /          |

|           -----           |

P0B |  1                    20  |

|                           |

P1B |  2                    19  | LM29B

|                           |

P0A |  3                    18  | HM29B

|                           |

P1A |  4                    17  | LM31B

|                           |

nRWS |  5                    16  | HM31B

|                           |

|  6                    15  | LM29A

|                           |

|  7                    14  | HM29A

|                           |

nLDS |  8                    13  | LM31A

|                           |

nUDS |  9                    12  | HM31A

|                           |

| 10                    11  | A15

|                           |

|                           |

-----------------------------

end of module _MS2_2

Page 1

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

Program Device Selector for MultiSync II, Location 185W

J. Margolin     13 January 1992

Equations for Module _MS2_3

Device MS2_3

- Reduced Equations:

!HM31A = (!A15 & !P0A & !P1A # P0A & !P1A & !nRWS & !nUDS);

!LM31A = (!A15 & !P0A & !P1A # P0A & !P1A & !nLDS & !nRWS);

!HM29A = (!P0A & !P1A & !nRWS & !nUDS # !A15 & P0A # !A15 & P1A);

!LM29A = (!A15 & P0A # !A15 & P1A # !P0A & !P1A & !nLDS & !nRWS);

!HM31B = (!BS0 & !P0B & !P1B # P0B & !P1B & !nRWS & !nUDS);

!LM31B = (!BS0 & !P0B & !P1B # P0B & !P1B & !nLDS & !nRWS);

!HM29B = (!P0B & !P1B & !nRWS & !nUDS # !BS0 & P0B # !BS0 & P1B);

!LM29B = (!BS0 & P0B # !BS0 & P1B # !P0B & !P1B & !nLDS & !nRWS);

Page 2

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

Program Device Selector for MultiSync II, Location 185W

J. Margolin     13 January 1992

Chip diagram for Module _MS2_3

Device MS2_3

P16V8S

----------\       /----------

|          \     /          |

|           -----           |

P0B |  1                    20  |

|                           |

P1B |  2                    19  | LM29B

|                           |

P0A |  3                    18  | HM29B

|                           |

P1A |  4                    17  | LM31B

|                           |

nRWS |  5                    16  | HM31B

|                           |

|  6                    15  | LM29A

|                           |

BS0 |  7                    14  | HM29A

|                           |

nLDS |  8                    13  | LM31A

|                           |

nUDS |  9                    12  | HM31A

|                           |

| 10                    11  | A15

|                           |

|                           |

-----------------------------

end of module _MS2_3

Page 1

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

CAS Decoder for MultiSync II, Location 135S

J. Margolin    4 November 1991

Equations for Module _MS2_4

Device MS2_4

- Reduced Equations:

!nCAS3 = (!LA25 & !LA26 & !PR & !nCASB & nRF

# DDOUT & !LA26 & !PR & !nCASB & nRF

# !nCASB & !nLAL & !nRF

# DDOUT & !LA26 & !nCASB & !nECAS & nRF

# LA25 & LA26 & LA4 & LA5 & !nCASB & nRF);

!nCAS2 = (!LA25 & !LA26 & !PR & !nCASB & nRF

# DDOUT & !LA26 & !PR & !nCASB & nRF

# !nCASB & !nLAL & !nRF

# DDOUT & !LA26 & !nCASB & !nECAS & nRF

# LA25 & LA26 & !LA4 & LA5 & !nCASB & nRF);

!nCAS1 = (!LA25 & !LA26 & !PR & !nCASB & nRF

# DDOUT & !LA26 & !PR & !nCASB & nRF

# !nCASB & !nLAL & !nRF

# DDOUT & !LA26 & !nCASB & !nECAS & nRF

# LA25 & LA26 & LA4 & !LA5 & !nCASB & nRF);

!nCAS0 = (!LA25 & !LA26 & !PR & !nCASB & nRF

# DDOUT & !LA26 & !PR & !nCASB & nRF

# !nCASB & !nLAL & !nRF

# DDOUT & !LA26 & !nCASB & !nECAS & nRF

# LA25 & LA26 & !LA4 & !LA5 & !nCASB & nRF);

!nCRAM = (LA24 & !LA25 & LA26 & !nCASB & nRF);

Page 2

ABEL(tm) 3.10  -  Document Generator             14-Jan-92 08:08 PM

CAS Decoder for MultiSync II, Location 135S

J. Margolin    4 November 1991

Chip diagram for Module _MS2_4

Device MS2_4

P16V8S

----------\       /----------

|          \     /          |

|           -----           |

nRF |  1                    20  |

|                           |

LA26 |  2                    19  |

|                           |

LA25 |  3                    18  | nCAS3

|                           |

nCASB |  4                    17  | nCAS2

|                           |

nECAS |  5                    16  | nCAS1

|                           |

DDOUT |  6                    15  | nCAS0

|                           |

LA5 |  7                    14  | LA24

|                           |

LA4 |  8                    13  | nCRAM

|                           |

nLAL |  9                    12  |

|                           |

| 10                    11  | PR

|                           |

|                           |

-----------------------------

end of module _MS2_4