TomCat Game Link Protocol

-------------------------

Each board set has an option switch to determine whether it is a master 'A' or a

slave 'B'. One board set should be selected as the master and the other board

set should be selected as the slave.

Each TX Buffer is normally off.

'A' can read the LNK Bus to receive data from 'B'.

'A' has LNKFLAG which is set when 'A' writes to its data latch. It is reset

when 'B' sends ACK. 'A' reads LNKFLAG to determine if 'B' has sent an ACK.

'A' has COMFLAG which is set when 'A' writes to its data latch with AB1=1.

COMFLAG is reset when 'A' writes to its data latch with AB1=0.

'B' can read the LNK Bus to receive data from 'A'.

'B' reads LNKFLAG and COMFLAG.

'B' sends ACK if there is no problem or ACK and ERR if there is.

--------------------------------------------------------------------------------

For 'A' to send a block of data to 'B':

'A':

1. Turns on its TX Buffer

2. Writes Comand to its TX Buffer (setting LNKFLAG and COMFLAG) and tells

'B' to expect a block of data (Perhaps Unit Vectors). It will tell it

how many words to expect and will do a checksum at the end.

'B':

1. LNKFLAG causes a LINK IRQ. It will stay in interrupt mode until the

transfer is complete.

2. Sees the LNKFLAG and the CMDFLAG

3. Reads the Command Data

4. Sends ACK (or ACK + ERR)

'A':

1. Waits for ACK

2. Sends next data word

'B':

1. Waits for LNKFLAG

2. Reads data

3. Sends ACK

Repeat until it has sent the block of data and the checksum.

If 'B's checksum matches, it will send ACK, otherwise it will send ACK + ERR.

'A' turns off its TX Buffer.

--------------------------------------------------------------------------------

For 'B' to send a block of data to 'A':

'A':

1. Turns on its TX Buffer

2. Writes Comand to its TX Buffer (setting LNKFLAG and CMDFLAG) and tells

'B' to send a block of data (Perhaps Unit Vectors). It will tell it

how many words to send and it will expect a checksum at the end.

'B':

1. LNKFLAG causes a LINK IRQ. It will stay in interrupt mode until the

transfer is complete.

2. Sees the LNKFLAG and the CMDFLAG

3. Reads the Command Data

4. Sends ACK (or ACK + ERR)

'A':

1. Waits for ACK

2. Turns off its TX Buffer

3. Does a dummy write to its data latch to set LNKFLAG (The latch output

is off).

'B':

1. Waits for LNKFLAG

2. Turns on its TX Buffer

3. Sends data

4. Sends ACK (to reset LNKFLAG)

'A':

1. Waits for ACK

2. Reads the Data

3. Does a dummy write to its data latch to set LNKFLAG (The latch output

is off).

'B':

1. Waits for LNKFLAG

2. Sends data

3. Sends ACK (to reset LNKFLAG)

Repeat until it has sent the block of data and the checksum.

'B' turns off its TX Buffer.

If 'A's checksum matches, it won't do anything. If not, it will request the

data again.

--------------------------------------------------------------------------------