TomCat

Hardware:

Tomcat was brought up in various forms on three different sets of hardware.

1.  Star Wars Hardware.

I don't remember if the code in Tc_StarWars is complete or if it is to

replace one or more ROMs in a standard Star Wars ROM set.

I think the version that runs on Star Wars is a helicopter.

2.  National Semiconductor 16032.

This was National Semiconductor processor (later renamed the 32016). It used

a National Semiconductor 16032 Evaluation Board somehow interfaced to a Star

Wars vector generator. I doubt the hardware still exists.

The National Semiconductor Evaluation Board was especially nasty because:

a)  It was connected to a VAX and all keystrokes from the keyboard were routed

through the VAX.

b)  It lacked the ability to move the cursor to change a command. If you

reached the end of a command and discovered an error at the beginning you had

to delete everything until you reached the character you wanted to change.

(You deleted that character and then typed everything over again.)

3.  The real 68010 TomCat hardware.

It used a 68010, a Texas Instruments TMS32010, and a 6502 Sound section.

It had the ability to link two games together using a 16-bit parallel

bidirectional interface with full handshaking. The software that was written

supports the Game Link.

Although the hardware supports using the 68010 and the TMS320 in parallel this

was not implemented. I thought I would leave the Programmers something to do.

It uses the standard Star Wars Control Yoke.

The Sound Group completely failed to support the project. Whatever exists runs

only with a White Box development system and requires hand patching to run.

No Sounds were developed by the Sound Group for TomCat.

Software:

You take off from a runway and fly a Jet around a grid. You can land, sort of.

The runway has Rabbit Lights.

There is another aircraft that you can shoot down. It does not shoot back. In

the linked game, this is the other Player and I think it can shoot back.

I have included:

1.  The software from the original VAX files. The 68010 files were assembled

with the VAX version of the Greenhills Assembler.

2.  I think I converted the 68010 source code to be assembled with the PC

version of the Greenhills Assembler.

3. I don't remember if the TMS320 Assembler had a version that ran on the VAX

or if it was always assembled with the PC version of the assembler.