320 Parallel Interface with Full Handshaking		J. Margolin  3/7/90

--------------------------------------------

1. The 68010 writes to a latch. This sets a Flag (68FLAG) that both 68010

and 320 can read. The 68FLAG is connected to the 320's BIO input for fast

data transfers. 68FLAG also generates a maskable 320 interrupt.

When the 320 reads the latch, 68FLAG is reset. (This is how the 68010

knows the 320 has read the latch.)

When the 68010 writes to the latch, an additional bit is latched (CMD).

This bit is read separately from the other 16 bits by the 320 and reading

it does not reset 68FLAG. This bit is connected to a 68010 address line,

and can be used to signify that the data in the latch is to be

interpreted as a command.

2. The 320 also has a data latch which, when written to, sets a flag

(320FLAG) that both 68010 and 320 can read. It does not generate a

68010 interrupt.

When the 68010 reads the latch, the 320FLAG is reset, which is how the

320 knows the data has been read.

3. There is an additional flag (XFLAG) that the 320 controls and the 68010

reads that is separate from the rest of the interface.

The 320P15 (aka PLD65) is located on the DSK PC Board.

The DSK Board contains:

DSP32C (aka PLD61)

320P15 (aka PLD65)

two 27C512s on a Slapstic

two 27C512s not on a Slapstic

two 8K*8 RAMs

two 2K*8 ZeroPower RAMs with write protect

DSKPEN		EQU	$85C000		* DSP32C Host Interface

DSKLATCH	EQU	$85C800

DSK_ZWDIS1	EQU	DSKLATCH+$04	* ZeroPower Write Disable 1

DSK_ZWEN2	EQU	DSKLATCH+$06	* ZeroPower Write Enable 2

DSK_320_RL	EQU	DSKLATCH+$08	* 320 Reset Low

DSK_LED_ON	EQU	DSKLATCH+$0E	* Led On

DSK_ZWEN1	EQU	DSKLATCH+$14	* ZeroPower Write Enable 1

DSK_ZWDIS2	EQU	DSKLATCH+$16	* ZeroPower Write Disable 2

DSK_320_RH	EQU	DSKLATCH+$18	* 320 Reset High

DSK_LED_OFF	EQU	DSKLATCH+$1E	* LED Off

To Enable Writes to the ZeroPower RAM:	CLR.W	DSK_ZWEN1

CLR.W	DSK_ZWEN2

To Disable Writes to the ZeroPower RAM:	CLR.W	DSK_ZWDIS1

CLR.W	DSK_ZWDIS2

XRAM8		EQU	$904000		* 16K Bytes

XZRAM		EQU	$910000		* 4K Bytes

XROM0		EQU	$920000	     * four 16K Bytes segments through Slapstic

XROM1		EQU	$940000		* 128K Bytes

68010-to-320 Host Interface

---------------------------

68WR 		EQU	$914000		* Write, CMD Flag = 0

68WR_CMD 	EQU	$914002		* Write, CMD Flag = 1 (command)

68RD0		EQU	$914000		* Read

68RD1 		EQU	$918000		* Read

d15 = 68FLAG

d14 = 320FLAG

d13 = /RSEL		not used

d12 = XFLAG