TomCat Rev 1	Jed Margolin

========================================

Changes and Corrections (PART 1) 12/26/85  :

Sheet 2

-------

Connect 3 A/B pin 4 to 3 A/B pin 8 (schematic only; board is ok.)

Sheet 3

-------

Rename from /WEH to /WEU and connect as required. (schematic, board)

Sheet 7

-------

7F/H gate on pins 1 and 2 should be removed and the line from 5F pin 14

(and R17) should go directly to /LINKIRQ . (schematic, board)

Sheet 9

-------

Gate 6U:	12 MHz should go to CK (pins 4 and 12)	(schematic, board)

/STOP should go to /K (pins 3 and 13)	(schematic, board)

8L and 8M have A and B inputs reversed. The best fix is to remove 8L pin 1

and 8M pin 1 from SA and connect it to /SA . (schematic, board)

Sheet 10

--------

Gate 10U:	Connect as per updated schematic (schematic, board)

Sheet 11

--------

R128: 	The end labled ZREF should be changed to XREF.(schematic, board)

Sheet 12

--------

Connect 14U/V and 14P to +15V and -15V. (There does not appear to be any room

for the bypass capacitors, so I think we will leave them off.)

Pin 13 is currently connected to +5V. This MUST be removed. <<<<<<<<<<<<<<<<<<<<<<<

Sheet 16

--------

Change all 74ALS244 to 74LS244	(schematic, parts list, assembly)

Change all 74ALS245 to 74LS245	(schematic, parts list, assembly)

Change all 8168-70  to 1420P55	(schematic, parts list, assembly)

Other changes to PC Board:

1. Move /WDDIS blade away from 6 MHz blade and give /WDDIS its own

ground blade.

2. Invert X and Invert Y should be normally open. (They are not.)

3. Move 68010 over a little to provide more clearance from the

tooling/mounting hole.

4. On the silkscreen, the ramps are shown on the wrong side of

all the in-line connectors.

================================================================================

TomCat Changes and Corrections (PART 2) 1/10/86 :

Sheet 2

-------

Install new Interrupt circuitry

Sheet 7

--------

5F is not sectioned properly

6E: Remove LNKFLAG from pin 9 and connect it to pin 8

Sheet 12

--------

Show +15v, -15v, and Ground connections on LF13201

Sheet 13

--------

11F/H: Add /SNMI to pin 6

Sheet 14

--------

10B is not sectioned properly

8D: Add /SNMI to pin 6

Sheet 15

--------

13A: Delete connection to pin 2

15C/D: Pin 1 is connected to a R147 instead of YAM2.

15A/B: Pin 1 is connected to R147 instead of R61.

Sheet 16

--------

74LS04 is not a Schmitt Trigger input.

U7406 is not a Schmitt Trigger input.

74AS04 is not a Schmitt Trigger input.

Assembly Mods to Rev 1 Boards:  (20+ cuts and 22+ jumpers)

Note: On TomCat, unused inputs are pulled up. On mods that use

previously unused inputs, these inputs must be disconnected

from the pullup line.

1). 	Connect wire jumper from 5E pin 3 to 2F pin 21.

2). 	Cut trace going to 8M pin 1, leaving intact the section that

goes to 8L pin 1.

Connect a wire jumper from 8M pin 1 to 7T pin 3

3). 	Cut 10U pin 15 free.

Connect a wire jumper from 10U pin 15 to 8U pin 1.

4). 	On 6U, swap pins 3 & 4 and also pins 12 & 13.

5). 	Lift the ZREF end of R128.

Connect a wire jumper from the free end of R128 to 13T/U pin 14.

6). 	14P: Lift pin 13;

Connect wire jumper from 14P pin 13 to 14M/N pin 8;

Connect wire jumper from 14P pin 4 to 14M/N pin 4.

7). 	14U/V: Lift pin 13;

Connect wire jumper from 14U/V pin 13 to 14T/U pin 8;

Connect wire jumper from 14U/V pin 4 to 14T/U pin 4.

8). 	7F/H: Lift pins 1 and 2;

Connect a wire jumper between pins 1 and 2 of the 7F/H footprint.

9). 	10B: Lift pins 7, 9, and 11;

Connect a wire jumper from the pin 11 footprint to pin 4;

Connect a wire jumper from the pin 9 footprint to pin 16.

10). 	New IRQ stuff:

Cut and lift 3C pin 13;

Cut and lift 3B pins 5, 9, and 11;

Cut trace going to 2A pin 24;

Cut trace going to 2A pin 25;

Connect a wire jumper from 5F pin 14 to 3C pin 13;

See schematic on sheet 2.

11). 	5F: Lift pin 16;

Connect a wire jumper from the pin 16 footprint to pin 5;

Connect a wire jumper from pin 4 to pin 15.

12).   6E: Lift pin 9;

Connect a wire jumper from the pin 9 footprint to pin 8.

13).   13A: On bottom of board, cut trace going to pin 2.

14).   Connect a wire jumper from 8D pin 6 to 11F/H pin 6.

15).   15C/D: Lift pin 1;

Connect a wire jumper from pin 1 to pin 2.

16).   15A/B: Lift pin 1;

Connect a wire jumper from 15A/B pin 1 to 14A pin 13.