TTL 'MAIN'

INCLUDE '[MARGOLIN.TC68]COMN2.ASM'

INCLUDE	'[MARGOLIN.TC68]EQUATES.ASM'

XDEF MULT1,MULT2,ANS,PXP,PYP,PZP,SX,SY

XDEF TEST_A,TEST_B,TEST_C,TEST_D,TEST_E,TEST_F,TEST_G

XREF D_CUBE,FASTCUBE

ORG $2400

* RAM

MULT1	DC.W	$4000

MULT2	DC.W	$4000

ANS	DS.W	1

PXP	DC.L	$10000000

PYP	DC.L	$10000000

PZP	DC.L	$10000000

SX	DS.W	1

SY	DS.W	1

PAGE

*Test time to format vectors

*D_CUBE was old method using subroutine calls (in DRAW2.ASM)

*Slow Cube = 380 uS for 16 lines = 23.8 uS/line

*Drawing routine is 156 bytes = 28 bytes overhead + 8 bytes/line (excluding

*   DRAW and PLOT routines which are common to all object routines)

* The Rocket with 51 lines takes up 436 bytes and would be expected to take

*   1.213 ms to format.)

TEST_A	MOVE.L	#$3000,A7

ALP1	MOVE.L	#$80000,VG_PTR

MOVE.B	D0,LED1ON

JSR	D_CUBE

MOVE.B	D0,LED1OFF

BRA	ALP1

*Test time to format vectors

*FASTCUBE was inline method, pre MACRO

*Fast Cube = 235 uS for 16 lines = 14.7 uS/line

*Drawing routine is 468 bytes = 28 bytes overhead + 28 bytes/line

*  The Rocket with 51 lines would be expected to take up 1456 bytes and take

*  749 uS to format, saving 464 us in exchange for 1020 bytes of memory.)

TEST_B	MOVE.L	#$3000,A7

BLP1	MOVE.L	#$80000,VG_PTR

MOVE.B	D0,LED1ON

JSR	FASTCUBE

MOVE.B	D0,LED1OFF

BRA	BLP1

* MULTIPLY TEST - SCALING  [4000 * 4000 = 1000 0000]

TEST_C	MOVE.W	MULT1,D0

MULS	MULT2,D0

MOVE.L	D0,ANS

BRA	*

* MULTIPLY TEST - DO 10 AND MEASURE FLAG

* 72 us/10 = 7.2 us/muls which is corect.

* 42 cycles at 6 MHz = 7 uS.

TEST_D	MOVE.B	D0,LED1ON

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MULS	D0,D1

MOVE.B	D0,LED1OFF

BRA	TEST_D

* DIVIDER TEST - Signed

* D0 Contains PXP (16 bits, max value will be 4000)

* From MACC it will be 1000 0000. Just swapping words will lose precision.

* For:	DIVS  	D0,D1	D1 = D1/D0

*			Inputs: D0 is 16 bits,

*				D1 is 32 bits

*			Output: D1 is 32 bits.

*				Low word is quotient,

*				High word is remainder.

*				(Appears to do 16 divide cycles)

*

* Examples:		   8/4	  = 2

*			4000/1	  = 4000

*			8000/2	  = 4000	(8000 is positive)

*			4000/4000 = 1

*		      4 0000/4000 = 10

*		     40 0000/4000 = 100

*		    400 0000/4000 = 1000

*		    800 0000/4000 = 2000

*		   1000 0000/4000 = 4000

*		   1FFF 0000/4000 = 7FFC

*		   2000 0000/4000 = 0000 R2000 (8000 Overflow?)

*		   2000 0000/5000 = 6666 R2000

*		   4000 0000/4000 = 0000 R4000 (8000 Overflow?)

*		   8000 0000/4000 = 0000 R8000 (8000 Overflow?)

TEST_E

DIVS	D0,D1

BRA	TEST_E

* DIVIDER TEST - Unsigned

* Examples:		   8/4	  = 2

*			4000/1	  = 4000

*			8000/2	  = 4000	(00008000 is positive)

*			4000/4000 = 1

*		      4 0000/4000 = 10

*		     40 0000/4000 = 100

*		    400 0000/4000 = 1000

*		    800 0000/4000 = 2000

*		   1000 0000/4000 = 4000

*		   1FFF 0000/4000 = 7FFC

*		   2000 0000/4000 = 8000

*		   2000 0000/5000 = 6666 R2000

*		   4000 0000/4000 = 0000 R4000

*		   8000 0000/4000 = 0000 R8000

*		   3FFF 0000/4000 = FFFC

TEST_F

DIVU	D0,D1

BRA	TEST_F

* DIVIDER TEST - DO 10 AND MEASURE FLAG

* 200 us/10 DIVS = 20 us/DIVS which is correct.

* 122 clock cycles at 6 MHz = 20.3 us. This is really too slow.]

TEST_G	MOVE.L	#$3000,A7

GLP1	MOVE.B	D0,LED1ON

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

DIVS	D0,D1

MOVE.B	D0,LED1OFF

BRA	GLP1

* [Time=65.5 uS]

* Projection 1		SX=K*YP/XP, SY=K*ZP/XP

TEST_H	MOVE.L	#$3000,A7

HLP1	CLR.W	LED1ON		*Timing Mark on

MOVE.L	PXP,D0		*D0 = 1000 0000   (PXP from MACC)

ASR.L  	#8,D0

ASR.L  	#6,D0	 	*D0 = 4000

MOVE.L	PYP,D1		*D1 = 1000 0000   (PYP from MACC))

DIVS  	D0,D1	 	*D1 = D1/D0 = 4000 is too big

ASR.W  	#5,D1		*D1 = 200 (VG will normalise it)

MOVE.W	D1,SX

*	(D0 = 4000)

MOVE.L	PZP,D2		*D2 = 1000 0000   (PZP from MACC))

DIVS  	D0,D2	 	*D2 = D2/D0 = 4000 is too big

ASR.W  	#5,D2		*D2 = 200 (VG will normalise it)

MOVE.W	D2,SY

CLR.W	LED1OFF		*Timing Mark off

BRA	HLP1

* [Time=61 uS]

* Projection 2		SX=K*YP/XP, SY=K*ZP/XP

TEST_I	MOVE.L	#$3000,A7

ILP1	CLR.W	LED1ON		*Timing Mark on

MOVE.L	PXP,D0		*D0 = 1000 0000   (PXP from MACC)

ASL.L 	#2,D0

SWAP 	D0

MOVE.L	PYP,D1		*D1 = 1000 0000   (PYP from MACC))

DIVS  	D0,D1	 	*D1 = D1/D0 = 4000 is too big

ASR.W  	#5,D1		*D1 = 200 (VG will normalise it)

MOVE.W	D1,SX

*	(D0 = 4000)

MOVE.L	PZP,D2		*D2 = 1000 0000   (PZP from MACC))

DIVS  	D0,D2	 	*D2 = D2/D0 = 4000 is too big

ASR.W  	#5,D2		*D2 = 200 (VG will normalise it)

MOVE.W	D2,SY

CLR.W	LED1OFF		*Timing Mark off

BRA	ILP1

* [Time=62 uS]

* Projection 3

* Inverse Multiplies  	SX=(K/XP)*YP, SY=(K/XP)*ZP

* (Scaling needs work, but it is close enough for timing comparison.)

TEST_J	MOVE.L	#$3000,A7

JLP1	CLR.W	LED1ON		*Timing Mark on

MOVE.L  #$800000,D1

MOVE.L	PXP,D0		*D0 = 1000 0000   (PXP from MACC)

ASL.L	#2,D0

SWAP	D0	 	*D0 = 4000

DIVS	D0,D1		*D1=80 0000/PXP    D1=200

*	(D1=1/PXP)

MOVE.L	PYP,D2		*PYP=1000 0000

ASL.L	#2,D2		*D2=4000 0000

SWAP	D2		*D2=4000

MULS.W	D1,D2		*D2=D1*D2  = 200*4000 = 80 0000

ASL.L	#2,D2		*D2 = 200 0000

SWAP	D2		*D2=200

MOVE.W	D2,SX		*

MOVE.L	PZP,D2		*PZP=1000 0000

ASL.L	#2,D2

SWAP	D2		*D2=4000

MULS.W	D1,D2		*D2=D1*D2  = 200*4000 = 80 0000

ASL.L	#2,D2		*D2 = 200 0000

SWAP	D2		*D2=200

MOVE.W	D2,SY		*

CLR.W	LED1OFF		*Timing Mark off

BRA	JLP1

* [Time=60 uS]

* Projection 4

* Inverse Multiplies	SX=(K/XP)*YP, SY=(K/XP)*ZP

* (Scaling needs work, but it is close enough for timing comparison.)

TEST_K	MOVE.L	#$3000,A7

KLP1	CLR.W	LED1ON		*Timing Mark on

MOVE.L  #$800000,D1

MOVE.L	PXP,D0		*D0 = 1000 0000   (PXP from MACC)

ASL.L	#2,D0

SWAP	D0	 	*D0 = 4000

DIVU	D0,D1		*D1=80 0000/PXP    D1=200

*	(D1=1/PXP)

MOVE.L	PYP,D2		*PYP=1000 0000

ASL.L	#2,D2		*D2=4000 0000

SWAP	D2		*D2=4000

MULS.W	D1,D2		*D2=D1*D2  = 200*4000 = 80 0000

ASL.L	#2,D2		*D2 = 200 0000

SWAP	D2		*D2=200

MOVE.W	D2,SX		*

MOVE.L	PZP,D2		*PZP=1000 0000

ASL.L	#2,D2

SWAP	D2		*D2=4000

MULS.W	D1,D2		*D2=D1*D2  = 200*4000 = 80 0000

ASL.L	#2,D2		*D2 = 200 0000

SWAP	D2		*D2=200

MOVE.W	D2,SY		*

CLR.W	LED1OFF		*Timing Mark off

BRA	KLP1

END