TCPARTS.DOC		Started 7/9/85		Jed Margolin  11/15/85

TomCat Parts Count

Totals:	One Board = Main Processor + AVG + Math + Sound = 189

TomCat Packages

3 Terminal  =  2

8 pin Dip   =  5

14 pin Dip  = 48

16 pin Dip  = 44

18 pin Dip  =  1

20 pin Dip  = 52

24 pin Wide =  5

28 pin Wide = 24

40 pin Wide =  6

64 pin Humungous = 1

Total = 188

TomCat Parts Cost

68010				20

27256		$3.75 * 19 =	 71.25

* 27256		$6.50 * 19 =	123.50  (old)

2K*8 RAM	$1.08 *  4 =	4.32

8K*8 RAM	$5.   *  3 =	15

AM6012  	$6.   *  2 =  	12

8168-55 	$2.45 *  4 =	9.8

Gate Array	         	6

6502A			  	3.

6532A			  	3.

TMS-5220C		  	6.50

POKEY		$4.   *  2 =   	8.00

YM2151				15

YM3012				5

TMS-320				42 (quan 1K)	37.50 (quan 5K)

------

220.87

$1. * 151  =	151

------

$371.87

TomCat (512K) = $371.87

TomCat (256K) = $341.87

Device	GND    +5V

-----------------------

LS00     7	14

S00	 7	14

LS02	 7	14

S02	 7	14

AS04     7	14

LS04	 7	14

7406	 7	14

7407	 7	14

LS08	 7	14

S10	 7	14

LS11	 7	14

LS14	 7	14

LS20	 7	14

LS27	 7	14

LS32	 7	14

LS42	 8	16

AS74	 7	14

LS74	 7	14

LS86	 7	14

S86	 7	14

109	 8	16

LS138	 8	16

LS139	 8	16

LS157	 8	16

LS161	 8	16

LS164	 7	14

LS174	 8	16

LS175	 8	16

LS191	 8	16

LS194	 8	16

LS244	 10	20

LS245	 10	20

LS259	 8	16

S260	 7	14

LS273	 10	20

LS299	 10	20

LS374	 10	20

LS393	 7	14

ADC0809	 13	11

68010  	 16,33	14,49

27256	 14	28

82HS137	 9	18

82S129	 8	16

8168-70  10	20

6502A	 1,39	 7

6532A	 1	20

Pokey	 1	17

TMS-320	 10	30

2K*8 RAM 12	24

8K*8 RAM 14	28

AM6012

DAC-08

LF13201

TL082

TL084

TMS-5220C

YM2151

YM3012

VG Gate

Array

TomCat Part Numbers

Quantity  Atari PN	Generic PN	Cost (ea)   Availability    Inventory

------------------------------------------------------------------------------

(4)	37-74LS00	74LS00

(1)	37-74S00	74S00

(2)	37-74LS02	74LS0

(1)	37-74S02	74S02

(1)	137433-001	74AS04

137437-001	74F04 (substitute)

(3)	37-74LS04	74LS04

(1)	37-7406		7406

(1)	37-7407		7407

(2)	37-74LS08	74LS08

(1)	137236-001	74S10

(1)	137149-001	74LS11

(2)	37-74LS14	74LS14

(3)	37-74LS20	74LS20

(2)	37-74LS27	74LS27

(4)	37-74LS32	74LS32

(1)	37-74LS42	74LS42

(3)	137435-001	74AS74

137436-001	74F74 (substitute)

(6)	37-74LS74	74LS74

(1)	37-74LS86	74LS86

(1)	137002-001	74S86

(1)	37-74109	74109

(5)	137177-001	74LS138

(2)	37-74LS139	74LS139

(2)	37-74LS157	74LS157

(7)	37-74LS161	74LS161

(1)	37-74LS164	74LS164

(1)	37-74LS174	74LS174

(6)	37-74LS175	74LS175

(6)	37-74LS191	74LS191

(6)	37-74LS194	74LS194

(25)	37-74LS244	74LS244

(11)	37-74LS245	74LS245

(1)	37-74LS259	74LS259

(1)	37-74S260	74S260

(2)	37-74LS273	74LS273

(3)	137180-002	74LS299

(5)	37-74LS374	74LS374

(2)	37-74LS393	74LS393

(1)	137243-001	ADC0809

(1)	137414-001	68010

-------------------

(19)	137396-300	27256 (to be programmed) [512K program]    (Note 1)

or

(11)	137396-300	27256 (to be programmed) [256K program]

-------------------

(4)	137211-001	2K*8 RAM

(3)	137441-001	HM6264-P15, 8K*8 Static RAM

-002	 (substitute)

-003	 (substitute)

(1)	137336-001	82HS137 (to be programmed)

(1)	136021-109	Programmed 37-82S129	(Note 2)

(4)	137360-002	8168-70  4Kx4 RAM

(1)	90-6013		6502A

(1)	90-6018		6532A

(2)	C012294-01	Pokey

(1)	137434-001	TMS-32010  (Note 3)

(2)	137158-002	AM6012

(2)	137159-001	DAC-08

(2)	37-13201	LF13201

(5)	37-TL082CP	TL082

(4)	37-347		TL084

(1)	137179-001	VG Gate	Array

(1)	37-7815		7815

(1)	37-7915		7915

(1)	137308-002	TMS-5220C

(1)	137401-001	YM2151

(1)	137402-001	YM3012

================================================================================

Notes:

1. Programmed Part Numbers are not assigned until the board is layed

out and PC locations are assigned:

(19) Programmed 137396-300 (27256)	[512K program]

or

(11) Programmed 137396-300 (27256)	[256K program]

2. Programmed Part Numbers are not assigned until the board is layed

out and PC locations are assigned:

(1) Programmed 137336-001 (82HS137)

3. 137434-001   NMOS Digital Signal Processor, TMS-32010, Texas Instruments

[General Instrument is to be an alternate source; samples are on

the way]

GI rep is Sally McGrew, (408) 496-0844

New GI rep is Earl Porter

General Instrument Corporation

3080 Olcott St  Suite 230 C

Santa Clara, CA  95051

For 5K she expects about $30. ($37.50 for TI)

================================================================================

P/N           GENERIC        DESCRIPTION                                    MANUFACTURER                  VENDOR P/N

137441-001    {         IC,STATIC_RAM,8K_X_8,150_NSEC,ICC_=_80MA             TOSHIBA                       TMM2064P-15

AMD                           AM99C88-15PC

HITACHI                       HM6264-P15

137441-002    {         IC,STATIC_RAM,8K_X_8,150_NSEC,ICC_=_45MA         _   RCA                           CDM6264-3

137441-003    {         IC,STATIC_RAM,8K_X_8,150_NSEC,ICC_=_30MA         _   FUJITSU                       MB8464-15

------------------------------------------------------------------------------------------------------------------------------------

137360-001    {         IC,STATIC_RAM,4KX4,NMOS,55NSEC                       FUJITSU                       MB8168-55

INMOS                         IMS1420P-55

137360-002    {         IC,STATIC_RAM,4KX4,NMOS,70NSEC                       FUJITSU                       MB8168-70

================================================================================

For TMS320 system:

8168-70   RAM	2.10 x 4 = 8.40  	(5K - Marshall)

74ALS244	1.33 x 6 = 7.98		(10K - Arrow)

74ALS245	1.33 x 4 = 5.32 	(10K - Arrow)

----

$21.70

8168-55   RAM	2.45 x 4 = 9.80  	(Inventory)

74LS244		.25  x 6 = 1.50		(Inventory)

74LS245		.28  x 4 = 1.12		(Inventory)

----

$12.42

The savings is $9.28 by using fast RAM but cheap buffers.

================================================================================

From the information supplied by Jim Wallin and Mary Burnias:

Most ICs are available stock to 6 to 8 weeks.

The exceptions are:

74AS04	TI 2 months ($0.31), Signetics del 3/21 ($0.17)

74AS74	TI 3 months ($0.31)

74F74 	Signetics ? ($0.195)

74ALS244 TI 3 months ($1.13)	[designed out already]

TL084	Motorola 12 weeks (LF347 $0.28)

================================================================================

VG Gate Array	AMI #9430-004

Bob Rush (marketing)  738-4151

Frank Markaby (marketing)

Mike Mackaby (engineering) 554-2198, 246-0330

The Problem:

AMI has scrapped our part. Completely.

They threw out the masks,

They threw out the data base,

They no longer make the base Gate Array.

I suggested that I give them one to open up. They could regenerate the

routing list from which they could regenerate the logic diagram which

they could transfer to a new Gate Array.

Mike Mackaby (Engineer) said they couldn't do it. Even the data

sheet specification that we have is not sufficient. He stated

that the only thing to do is to start from scratch.

His estimate, for a person already experienced in working with

Gate Arrays is:

5 weeks design

3-4 weeks prototype

4-6 weeks production quantities

That comes to 12 - 15 weeks for an experienced person.

I am not experienced in doing Gate Arrays and I don't know

anyone at Atari who is.

The Solution (Choose one):

A. In the event they are telling the truth, we are stuck. There

are 1000 Gate Arrays in inventory and even they will probably be

lost or scrapped before we could get to them. We could do an

Oops! Board if we can get the register file ICs (We had problems

getting them before).

B. If they are not telling the truth (because they consider 5K pieces

as not being worth their time) it is a matter for one of our VPs

to take up with one of their VPs.

C. Scrap TomCat and wait for Sherman's hardware.