.TITLE	TCAVG 	;AVG STATE PROM FOR TOMCAT

.RADIX	16

.ASECT

.ENABL LC

; PROGRAMMER: JED MARGOLIN 1/21/85

; REV 1.0

A=0A

B=0B

C=0C

D=0D

E=0E

F=0F

X=0

;USES A 256X4 BIPOLAR PROM TO GENERATE STATES

;A7=NOT HALT & NOT GO

;A6=OP2

;A5=OP1

;A4=OP0

;A3=F3 (from Q3)

;A2=F2 (from Q2) Q2=ST2

;A1=F1 (from Q1)

;A0=F0 (from Q0)

;INSTRUCTIONS		Vector State Sequence

; 0	VCTR 		9,8,B,A,C,D,F,0			4.66 us + drawing time

; 1	HALT 		9,8,F,0				2.66 us

; 2	SVEC 		9,B,C,D,F,0			3.33 us + drawing time

; 3	STAT (old) 	9,8,E,0				2.66 us

; 3	STAT (new) 	9,8,7,6,5,E,0  (because of X,Y,Z delay)       4.66 us

; 4	CNTR	   	9,8,C,F,0			3.33 us + centering time

; 5	JSRL	   	9,8,C,D,E,9 (GA address provides new opcode)  4.00 us

; 6	RTSL	   	9,8,D,E,9   (GA address provides new opcode)  3.33 us

; 7	JMPL		9,8,E,9	    (GA address provides new opcode)  2.66 us

.REPT	80		;IF A7=0, ALWAYS STATE 0

.BYTE 0

.ENDR

;addresses    0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F

.BYTE 9,X,X,X,X,X,X,X,B,8,C,A,D,F,X,0	;VCTR-000

.BYTE 9,X,X,X,X,X,X,X,F,8,X,X,X,X,X,0	;HALT-001

.BYTE 9,X,X,X,X,X,X,X,X,B,X,C,D,F,X,0	;SVEC-010

;	.BYTE 9,X,X,X,X,X,X,X,E,8,X,X,X,X,0,X	;STAT-011 (old)

.BYTE 9,X,X,X,X,E,5,6,7,8,X,X,X,X,0,X   ;STAT-011 (new)

.BYTE 9,X,X,X,X,X,X,X,C,8,X,X,F,X,X,0	;CENTER-100

.BYTE 9,X,X,X,X,X,X,X,C,8,X,X,D,E,9,X	;JSR-101

.BYTE 9,X,X,X,X,X,X,X,D,8,X,X,X,E,9,X	;RTS-110

.BYTE 9,X,X,X,X,X,X,X,E,8,X,X,X,X,9,X	;JMP-111

.END

.NLIST

STATES: Q2 (ST2) = 0 for Memory Access Cycles

Q3 = 0 does not generate any strobes or latches, and is

used to generate wait states. (Forces ST3 high)

Q3  Q2  Q1  Q0

----------------

1   0   0   0	  (8)	LATCH 0		load DVY0-DVY7,

INC PC

1   0   0   1    (9)	LATCH 1		load DVY8-DVY15	(contains opcode),

Clear DVY0-DVY7, DVX3-DVX12, Z0-Z2,

INC PC

1   0   1   0    (A)  	LATCH 2		load DVX0-DVX7,

INC PC

1   0   1   1    (B)	LATCH 3		load DVX8-DVX15,

INC PC

1   1   0   0    (C)	STROBE 0

VCTR: Normalize, Set AVG0

SVEC: Normalize, Set AVG0

CNTR: Normalize, Set AVG0

JSRL: Write Data from Internal GA bus	to Register File

(Store PC on Stack), Set AVG0

1   1   0   1    (D)	STROBE 1

VCTR: Binary Scale, Set AVG0

SVEC: Binary Scale, Set AVG0

JSRL: Increment Stack Pointer, Set AVG0

RTSL: Decrement Stack Pointer, Set AVG0

1   1   1   0    (E)	STROBE 2

STAT: Do STAT (Z-SCALE or SCALE-LD), Set AVG0

JMPL: Load PC with DVY0-DVY12, Set AVG0

JSRL: Load PC with DVY0-DVY12, Set AVG0

RTSL: Load PC from Stack Output, Set AVG0

1   1   1   1    (F)	STROBE 3

VCTR: Start Vector, Set AVG0

SVEC: Start Vector, Set AVG0

CNTR: Start Center, Set AVG0

HALT: Set Halt, Set AVG0

AVG0 Low to High will increment AVG1-13.

VCTR Example (Bytes):		00  DYL

01  VCTR, DYH

02  DXL

03  Z, DXH

04  next data

05  next opcode, data

PC(Bin)  Q   ST3^? ST2

------  ---  ---   ---

0001    0    N     0

9    Y     0  Latch 1:

Load DVY8-DVY15 (contains opcode),

Clear DVY0-DVY7, DVX3-DVX12, Z0-Z2,

INC PC

0000	  8    Y     0	Latch 0:

Load DVY0-DVY7,

INC PC

0011	  B    Y     0  Latch 3:

Load DVX8-DVX15,

INC PC

0010    A    Y     0  Latch 2:

Load DVX0-DVX7,

INC PC

0101	  C    Y     1  Strobe 0: Normalize, Set AVG0

D    Y     1	Strobe 1: Binary Scale, Set AVG0

F    Y     1	Strobe 3: Start Vector, Set AVG0

0    N     0	Spin until Vector is done

0101  next instruction

------------------------------------------------------------------------------

VCTR Example (Words):		01  VCTR, DYH, DYL

00  Z, DXH, DXL

03  next opcode, data

PC(Bin)  Q   ST3^? ST2

------  ---  ---   ---

0001    0    N     0

9    Y     0	Latch 1 & Latch 0:

Load DVY8-DVY15 (contains opcode),

Clear DVY0-DVY7, DVX3-DVX12, Z0-Z2,

Load DVY0-DVY7,

INC PC

0000	  B    Y     0	Latch 3 & Latch 2:

Load DVX8-DVX15,

Load DVX0-DVX7,

INC PC

0011	  C    Y     1  Strobe 0: Normalize, Set AVG0

D    Y     1	Strobe 1: Binary Scale, Set AVG0

F    Y     1	Strobe 3: Start Vector, Set AVG0

0    N     0	Spin until Vector is done

0011	  9  next instruction

------------------------------------------------------------------------------

JSRL/RTSL Example (Bytes):	000	(04)

001	JSRL, (00) H  	(EA=08)

002	next data

003	next opcode

008	XX

009	RTSL

PC(Bin)  Q   ST3^? ST2

------  ---  ---   ---

0001    0    N     0

JSRL

9    Y     0	Latch 1, INC PC

0000	  8    Y     0	Latch 0, INC PC

0011	  C    Y     1 	Strobe 0: Store AVG1 - AVG13 (=0001) on Stack, Set AVG0

0011	  D    Y     1	Strobe 1: Increment Stack Pointer, Set AVG0

0011	  E    Y     1	Strobe 3: Load PC with DVY0-DVY12 (0100) * 2, Set AVG0

1001

RTSL

1001	  9    Y     0  Latch 1, INC PC

1000    8    Y     0  Latch 0, INC PC

1011	  D    Y     1  Strobe 1: Decrement Stack Pointer, Set AVG0

1011    E    Y     1  Strobe 2: Load PC from Stack (0001) * 2 , Set AVG0

0011    9  next instruction

JSRL/RTSL Example (Words):	0001	JSRL, (00), (04)	(EA = 08)

0000	next opcode, data

0003	next opcode, data

0002 	next opcode, data

0009	RTSL

PC(Bin)  Q   ST3^? ST2

------  ---  ---   ---

0001    0    N     0

JSRL

9    Y     0	Latch 1, Latch 0, INC PC

0000	  C    Y     1 	Strobe 0: Store AVG1 - AVG13 (=0000) on Stack, Set AVG0

0011	  D    Y     1	Strobe 1: Increment Stack Pointer, Set AVG0

0011	  E    Y     1	Strobe 3: Load PC with DVY0-DVY12 (0100) * 2, Set AVG0

1001

RTSL

1001	  9    Y     0  Latch 1, Latch 0, INC PC

1000    D    Y     1  Strobe 1: Decrement Stack Pointer, Set AVG0

1011    E    Y     1  Strobe 2: Load PC from Stack (0000) * 2 , Set AVG0

0001    9  next instruction, oops! wrong address.

A. JMPL and JSRL use (address * 2) + 1 and always goes to an odd address.

Because of how AVG0 is handled, it always returns to an odd address.

B. The Stack does not remember AVG0. In a word mode, the return address

it may be either odd or even.  In Byte mode it is always odd.

C. For an average vector of Vmax/2 = 90 uS, Word architecture would save

1.33 uS or 1.5 %. Word architecture would make Short Vectors impossible

without a bunch more hardware.

D. It looks like the VG architecture will be Bytes, although the 68010 will

address it as words.

E. Although memory 'hits' with the 68010 will occur more often with a VG

byte architecture, they should still be rare.

F. Whew.

Notes:

Battlezone aligns LCLK with /1.5 MHz and stalls low.

Star Wars aligns LCLK with /1.5 MHz and stalls high.

Quantum aligns LCLK with /1.5 MHz and stalls high.

(VGCLK & /STROBE 3) is the window for the last 12 MHz clock in

the cycle.

VG Memory Contention:

6502/6809:

The 6502/6809 are synchronous processors, as is the State Machine.

A memory access is always initiated at the same point of the 1.5 MHz clock. If

both the processor and the State Machine start a VG memory access, the

processor has priority and State Machine is stalled for that cycle.

When the State Machine is doing a memory access, ST2 is low. If ST2 is

low during /VMEM, the State Machine is stalled. However, ST2 is also low

during CENTER and VECTOR execution. Stalling the State Machine during this time

does not affect the execution of the Vector or Center instructions; the State

Machine would otherwise be spinning.

68010:

This is an asynchronous processor; the memory access can begin during

any 6 MHz clock.

If the State Machine is not accessing memory, the 68010 can have

the bus with no wait states.

If the 68010 is not accessing memory, the State Machine can have the

bus with no stall cycles.

If the State Machine is in the middle of a memory access (LCLK Low),

the 68010 will have to wait if it wants memory.

If the 68010 is accessing memory at the beginning of a State Machine

memory cycle, the State Machine will have to stall for that cycle.

Because ST2 is low during CENTER and VECTOR executions as well as

memory accesses, it would make the 68010 wait unnecessarily. If it made it

wait two extra 6MHz periods (333 ns) and the vector memory was divided into

two 8K byte buffers (4K words), it would waste 1.3 ms of 68010 time. Even

0.6 ms (4K bytes=2K words) is too much.

When the State Machine is stalled by a VECTOR or CENTER execution,

all of the Q states are low. (Q3 low prevents ST3 from producing Latch or

Strobe pulses.) The State Machine is doing a memory access only during

ST2 low and Q3 high. Both are stable 94 ns into LCLK high.

TomCat AVG Timing (13.9 ns/div)

@@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@

12 MHz     @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @

@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@

@@    @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@

6 MHz	 @    @     @     @     @     @     @     @     @     @     @     @

(LS161) @@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@

@@	   @@@@@@@@@@@@@           @@@@@@@@@@@@@           @@@@@@@@@@@@@

3 MHz	@	   @  	       @	   @	       @	   @

@@@@@@@@@@@@           @@@@@@@@@@@@@           @@@@@@@@@@@@@

@@	               @@@@@@@@@@@@@@@@@@@@@@@@@

1.5 MHz	@  		       @		       @

@@@@@@@@@@@@@@@@@@@@@@@@                       @@@@@@@@@@@@@@@@@@@@@@@@

@@@@@@@@@@@@@@@@@@@@@@@@@   		      @@@@@@@@@@@@@@@

LCLK, ST3     @			      @	 Latches, Strobes     @

(LS74)	@@@@@@@			      @@@@@@@@@@@@@@@@@@@@@@@@@

LS174=30, LS157=14, PROM=50, Total=94ns

| 94  |   Q valid              		      |

ST3 low to high increments GA address counter

| 168 ns 	  | GA address valid		       |

Stalls high

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

@			      @	 		      @

LCLK, ST3     @			      @	 Latches, Strobes     @

(LS74)	@@@@@@@	  stall before >|     @@@@@@@@@@@@@@@@@@@@@@@@@

------------------------------------------------------------------------------

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

_____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

________             ___________             ___________             __

3 MHz	        \___________/           \___________/           \___________/

________                         _______________________

1.5 MHz	        \_______________________/                       \______________

_______________________   Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/

stall before >|

| 94  |   Q valid              		      |

| 168 ns 	  | GA address valid		      |

ST2 low and Q3 high. Both are stable 94 ns into LCLK high.