ATARI GAMES CORP.						Page 1 of 7

Specifications for Vector Generator Gate Array

Jed Margolin 	4/21/86

(408) 434-1730

The Gate Array is designed to be the Stack/Stack Pointer and Program

Counter for the Analog Vector Generator used in vector type games.

The Program Counter has 14 bits. The Stack is four levels deep and

stores the 14 bit Program Counter. When the Gate Array receives control

signals from the Analog Vector Generator it executes the JMP, JSR, and RTS

instructions.

1. The normal Gate Array operation is to increment the Program Counter

(INC PC) which is controlled by ST2 and ST3 supplied by external hardware.

2. For the JMP instruction, /STROBE 2 loads the PC with the address

data on DVY0 - DVY12.

3. For the JSR instruction, /STROBE 0 stores the current PC address onto

the Stack, /STROBE 1 increments the Stack Pointer, and /STROBE 2

loads the PC with the address data on DVY0 - DVY12.

4. For the RTS instruction, /STROBE 1 decrements the Stack Pointer and

/STROBE 2 loads the PC from the Stack Outputs.

Note that the JMP and JSR instructions go to an address with the least

significant bit zero with the remaining 13 bits supplied by DVY0 - DVY12 .

An RTS instruction will cause the PC to return to the appropriate address,

whether even or odd.

Page 2 of 7

--------------------

|			   |

AVG3 -| 1		40 |- AVG2

AVG4 -| 2		39 |- AVG1

AVG5 -| 3		38 |- ST3

/STROBE 2 -| 4		37 |- VDD

/OP2 -| 5		36 |- AVG0

/VGGO -| 6		35 |- ST2

DVY3 -| 7		34 |- /STROBE 1

DVY4 -| 8		33 |- OP1

DVY5 -| 9		32 |- DVY0

DVY6 -| 10		31 |- DVY1

DVY7 -| 11		30 |- DVY2

DVY8 -| 12		29 |- DVY10

DVY9 -| 13		28 |- DVY11

VSS -| 14		27 |- DVY12

AVG6 -|	15		26 |- /OP0

AVG7 -|	16		25 |-

AVG8 -|	17		24 |- /STROBE 0

AVG9 -|	18		23 |-

AVG10 -|	19		22 |- AVG13

AVG11 -|	20		21 |- AVG12

|			   |

--------------------

Figure 1.   Pin Connections

Page 3 of 7

DC Characteristics at: TA = 0 to 70 deg Celsius, VDD = 5V +/- 5%

PARAMETER	SYMBOL	   CONDITIONS		MIN	MAX	UNITS

---------	------	   ----------		---	---	-----

High-Level

Input Voltage	VIH				2	VDD	V

Low-Level

Input Voltage	VIL				0	0.8	V

High-Level

Output Voltage	VOH	    VDD = 5.00		2.7		V

IOH = -700 uA

VIN = VSS or VDD

Low-Level

Output Voltage	VOL	    VDD = 5.00			0.4	V

IOL = 0.8 mA

VIN = VSS or VDD

Input

Capacitance	CIN					5	pF

Page 4 of 7

Parameter			Symbol		Min	Max	Units

--------			-----		---	---	----

Propagation Delay

/VGGO to

AVG0 - AVG13			Tvggd			168	ns

/VGGO Pulse

Width				Twvg		120		ns

Cycle Time  			Tscyc		500		ns

Set up				Tsu		20		ns

Hold				Thld		20		ns

Propagation delay		Tst3d			168	ns

ST3 to AVG0 - AVG13

OPCODE Set-up time to

/STROBE 0, 1, and 2		Tsuoc		200		ns

/STROBE 0, 1, and 2

Pulse Width			Tpst		150		ns

Propagation delay

/STROBE 2 low to

AVG0 - AVG13			Tst2d			168	ns

DVY Set-up time to

/STROBE 0, 1, and 2		Tsud		200		ns

DVY Hold time after

/STROBE 0, 1, and 2		Thdd		200		ns

OPCODE Hold Time after

/STROBE 0, 1, and 2		Thdoc		200		ns

Delay Time Between

/STROBE 1, 2, and 3		Tssd		150		ns

Page 5 of 7

Timing Requirements for Vector Generator Gate Array

---------------------------------------------------

Reset:

-----

| --Twvg-- |

___________________	       _________________________________________

/VGGO			   \__________/

|	|

>|	|< Tvggd

|	|

AVG1     _______________________

- AVG13				\______________________________________________

|	|

|	|

>|	|< Tvggd

|	|_______________________________________________

AVG0     _______________________/

INC PC:

------

|---- Tscyc ---------|

__________		 __________	       __________

ST3	__________/	     \__________/	   \__________/	         \__

|     |		|    |

Tsu >|     |<	       >|    |< Thld

|________________|___ |

ST2	_______________/		|   \______________________________

|

|

|

________________________________  _____________________________

|   \/

AVG0-AVG13  ________________________________/\_____________________________

|   |

>|   |< Tst3d

|   |

Page 6 of 7

JMP:

_________		        ____________________________

/OP0,/OP2		 \_____________________/

Tsuoc	>|	|<    >|       |< Thdoc

|	|      |       |

|      |

_________  _____|______|_______  ___________________________

DXY0-12			 \/	|      |       \/

_________/\ ____|______|_______/\___________________________

|	|      |       |

|<Tsud>|      |<Thdd >|

|      |

/STROBE 2	________________|      |____________________________________

\______/

|      |

|-Tpst-|

|      |

________________|__  __|____________________________________

AVG0 - AVG13 			|  \/  |

________________|__/\__|____________________________________

|

>|  |< Tst1d

|  |

RTS:

___________________________________________________

/OP0,OP1	_________/      |

>|	|< Tsuoc

|

_________	|

/OP2			 \______|____________________________________________

>|	|< Tsuoc

|

/STROBE 1	________________|	 ____________________________________

\_______/

|	|

|-Tpst-	|

|	|

|	|-Tssd-	|

/STROBE 2	________________________|_______	 ____________________

|	\_______/

|	|

|	|-Tpst- |

|

________________________________|___  _______________________

AVG0 - AVG13 					|   \/

________________________________|___/\_______________________

|    |

>|    |< Tst2d

|    |

Page 7 of 7

JSR:

_________		        		        ______

/OP0,OP1,/OP2		 \_____________________________________________/

Tsuoc	>|	|<    			      >|       |< Thdoc

|	|      			       |

|             		       |

______________________________  ________________________  ___

DXY0-12		      		              \/	       |        \/

______________________________/\ _______________________/\___

|		|      	|      |

|		 <Tsud>	|      |<Thdd >|

|      |

/STROBE 0	________________|      |______________________________________

\______/		|      |

|      |		|      |

|-Tpst-| 		|      |

|Tssd|	        |      |

/STROBE 1	____________________________|       __________________________

\______/    |      |

|      |    |      |

|-Tpst-|    |      |

|Tssd|      |

|      |

/STROBE 2	________________________________________|       ______________

\______/

|      |

|-Tpst-|

|

________________________________________|__  _________________

AVG0 - AVG13 						|  \/

________________________________________|__/\_________________

|

>|  |< Tst2d

|  |

Appendix

4/25/86	  Received a call from Todd Goldsmith from Norcomp who is doing

the gate count estimate. (727-7707)

1. With the Fujitsu process, VIH is 2.2 V (instead of AMI 2.0 V). To make

certain the inputs are LSTTL compatible I asked him to add pullups to all

inputs. Pullups are 50K typical.

2. VOH is changed from:

High-Level

Output Voltage	VOH	    VDD = 5.00		2.7		V

IOH = -700 uA

VIN = VSS or VDD

To:

High-Level

Output Voltage	VOH	    VDD = 4.75		4.2		V

IOH = -400 uA

VIN = VSS or VDD

which is an improvement.

3. Maximum input capacitance increases from 5 pF. to 8 pF. which is

no problem.

JM

================================================================================

4/30/86	  Received a call from Todd Goldsmith from Norcomp who is doing

the gate count estimate. (727-7707)

He asked for permission to use pin 23 (CARRY) or pin 25 (OPX)

for an additional ground. I said yes.

I do not use the CARRY output and to my knowledge no one ever

has. OPX duplicates OP0. The TomCat PC can be easily changed.

In the event this part is used as a replacement in games that used

the AMI parts we can include instructions to connect a jumper from pin 23

(CARRY) to ground. If pin 25 (OPX) ends up being used, then they will

cut the trace going to that pin and then ground the pin.

------------

His gate count comes to 802. They have a gate array with 850 gates

(the 850 AVB). If it ends up not fitting they have one with 1200 gates

(1200 AVB).

5/12/86	  Offer from Norcomp for Fujitsu gate array.

A. Initiation of Program		$10,000.

B. Schematic Conversion			included

C. Simulation/Layout/Validation		$4,080.

D. Prototypes and final approval	$5,450.

E. Additional samples (max 650 pcs)	$46.

F. Qualification samples (max 250 pcs)	$120.

Production:	 1K	 5K

----	-----

$4.90	$4.60

Contract says that the buyer is responsible for the test vectors.

Therefore: 1K = 19,530 NRE + 4,900 = 24,430 which is $24.43 each.

5K = 19,530 NRE + 23,000 = 42,530 which is $8.50 each.

For oops board = $13 (approximately)

Breakeven Point:

n * 13 = 19,530 + (n * 4.90)

(n * 13) - (n * 4.90) = 19,530

n * 8.10 = 19,530

n = 2,411 pcs

Even if the oops board costs $18

Breakeven Point:

n * 18 = 19,530 + (n * 4.90)

(n * 18) - (n * 4.90) = 19,530

n * 13.10 = 19,530

n = 1,490 pcs

The AMI quote was:

Design complete circuit from scratch in 3 micron or 2 micron

double metal gate array. We supply schematic and test vectors.

(Unfortunately they did not give a price for 1K quantity)

Lead time: 10 weeks

NRE: $13.5K

Unit Price: $3.46 (5K)

The breakeven point for a $13 oops board is

n * 13 = 13,500 + (n * 3.46)

(n * 13) - (n * 3.46) = 13,500

n * 9.54 = 13,500

n = 1415 pcs

Summary for Turnkey Gate Array (prices include NRE)

Atari would be responsible for supplying the

schematic and the Test Vectors.

1K	2.5K	 5K

---	----	----

Oops Board	$13.	$13.	$13.

Fujitsu		$24.43  $12.71	$8.50

AMI		  not quoted	 $6.16

If we only sell 1,000 the project is a flop and we don't need anybody's

Gate Array.

If we sell 5,000, AMI's Gate Array is the most cost effective.