*******************************************************************************

*AERODYNAMICS:

*

*	Indicated Airspeed (as measured by a Pitot tube) is the component

*	of velocity along the aircraft's X direction.

*

*	Parasitic Drag is opposite the velocity vector.

*	Parasitic Drag will be proportional to Velocity (instead of squared).

*

*	Gravity is down.

*

*	For an Airplane:

*		Thrust is along (AX,BX,CX)

*

*		The component of wind normal to the wing is

*		QW = (-VX*AZ -VY*BZ -VZ*CZ)

*

*		The force on the Wing is along (AZ,BZ,CZ)

*

*

*	AERO:

*		VELOCITY (FORCE) NORMAL TO THE WING: QW = -VX*AZ -VY*BZ -VZ*CZ

*

*		FX = THRUST*AX + PDR*VX + QW*AZ

*

*		FY = THRUST*BX + PDR*VY + QW*BZ

*

*		FZ = THRUST*CX + PDR*VZ + QW*CZ + GRAVITY

*

*

*		Update:	VX' = VX + K1*FX

*			VY' = VY + K1*FY

*			VZ' = VZ + K1*FZ

*

*			XT' = XT + K2*VX

*			YT' = YT + K2*VY

*			ZT' = ZT + K2*VZ

*

*

* QW * (AZ, BZ, CZ) represents the force due to the plane's angle of attack

* (the angle that the relative wind makes to the wing [chord]. In this model

* the relative wind is due entirely to the plane's velocity. Normally this

* force (which produces lift and drag) is proportional to the square of the

* velocity. Here it is linearly proportional to the velocity.

*

* Parasitic Drag is proportional to the square of the speed, but in this model

* will be linear.

*

* Lift and Drag is just a way of saying that if you look at the force

* produced on the wing, (and you are flying right-side-up with a reasonable

* angle of attack), the force will have a component opposite gravity (lift)

* and a component opposite the Plane's [straight ahead] motion (drag).

*

* If the Wings are not level, there will also be a component to the side,

* which by itself would produce side slip. However, the Tail acts as a

* weathercock to turn the Nose into the [relative] wind. This is why an

* airplane banks and doesn't just slip to the side.

*

*

*	For a Helicopter:

*		Thrust is along (AZ,BZ,CZ)

*

*		Treat the Rotor as a wing

*

*==============================================================================

* Another way of looking at parasitic drag is to consider the forces

* consisting of:

*

* 	FX = THRUST*AX + QW*AZ			[Engine + Wing]

*	FY = THRUST*BX + QW*BZ			[Engine + Wing]

*	FZ = THRUST*CX + QW*CZ - GRAVITY	[Engine + Wing + Gravity]

*

* To update the velocity:

* 	VX' = VX + K*FX - K*VX		(K*VX = Parasitic Drag)

* 	    = VX + (FX - VX) * K

*

* which converges at VX' = FX; K simply	determines how fast it converges.

*

* Similarly,

*	VY' = VY + (FY - VY) * K	Converges at VY = FY

*	VZ' = VZ + (FZ - VZ) * K	Converges at VZ = FZ

*

*==============================================================================

* The completely general case is where:	 [with new K1,K2]

*

* 	VX' = VX + K1*K2*FX - K2*VX	(K2*VX = Parasitic Drag)

*

*	    = VX + (FX*K1 - VX) * K2

*

* which converges at VX' = K1*FX;  K2 determines how fast it converges.

*

*===============================================================================

* Looking again at the original Aerodynamic equations, Let:

*

*		PDR = Coefficient of Parasitic Drag

*

*		FX = THRUST*AX + QW*AZ			[Engine + Wing]

*

*		FY = THRUST*BX + QW*BZ			[Engine + Wing]

*

*		FZ = THRUST*CX + QW*CZ + GRAVITY	[Engine + Wing]

*

*

*		Update:	VX' = VX + K*FX - PDR*VX

*			    = VX + [(K/PDR)*FX - VX] * PDR

*

*	The Velocity converges at K/PDR, PDR determines how fast it converges.

*

*	The others are:	VY' = VY + K*FY - PDR*VY

*			    = VY + [(K/PDR)*FY - VY] * PDR

*

*			VZ' = VZ + K*FZ - PDR*VZ

*			    = VZ + [(K/PDR)*FZ - VZ * PDR

*

********************************************************************************

PAGE

XDEF GRAV,MODE,TSCALE,TURNSC,POS,POS2,WING

XDEF UPOS,YWIND,YW3,VCOM,FSHIFT,FORCE,PSHIFT

MODE:	DC.W	$1		* 0=Airplane, 1=Straight, 2=Helicopter

TSCALE: DC.W	$4000		* Thrust scaling

GRAV:	DC.W 	$1000		* Gravity

TURNSC  DC.W	$28		* Turning Scale

FSHIFT	DC.W	$06		* FORCE SCALING (SHIFTS)

PSHIFT	DC.W	$09		* POSITION SCALING (SHIFTS)

FLY3:

* Crude Airplane Aerodynamics for Ship 1

* ROLL TIC

* D0 CONTAINS DELTA ANGLE FOR ROLL, +/- 79 MAX

MOVEA.W	#UV1,A0			*A0=Ship 1 primary

MOVE.B	POT1,D0			*Roll Pot

JSR	T_ROLL

* PITCH TIC

* D0 CONTAINS DELTA ANGLE FOR PITCH, +/- 79 MAX

MOVEA.W	#UV1,A0			*A0=Ship 1 primary

MOVE.B	POT0,D0			*Pitch Pot

JSR	T_PTCH

* Calculate Force due to Thrust. Distribute thrust along (AX,BX,CX)

FY3B	MOVEA.W	#UV2,A0		*UV2 is the Working Set

* Thrust

MOVE.W	THRUST(A0),D1

MULS	TSCALE,D1

ASL.L	#2,D1

SWAP	D1		*D1 = scaled thrust

MOVE.W	AX(A0),D0

MULS	D1,D0

ASL.L	#2,D0		* AX * THRUST, MAX = 4000 0000

SWAP	D0

MOVE.W	D0,FX1		* FX = THRUST*AX, MAX = 4000

MOVE.W	BX(A0),D0

MULS	D1,D0

ASL.L	#2,D0		* BX * THRUST, MAX = 4000 0000

SWAP	D0

MOVE.W	D0,FY1		* FY = THRUST*BX, MAX=4000

MOVE.W	CX(A0),D0

MULS	D1,D0

ASL.L	#2,D0		* CX * THRUST, MAX = 1000 0000

SWAP	D0

MOVE.W	D0,FZ1		* FZ = THRUST*CX, MAX=4000

WING:	BSR	VCOM		* Find Components of relative wind to ship

MOVE.W	XP,D0		* USE VXCOM FOR AIRSPEED

TST.W	D0

BPL	WNG2

NEG.W	D0

WNG2	SUB.W	ASPD,D0

ASR.W	#3,D0

ADD.W	D0,ASPD		* ASPD = (NEW-OLD)/8 + OLD

MOVE.W	ZP,D1		* D1= - VZCOMP = QW  (Force on wing)

MOVE.W	AZ(A0),D0

MULS	D1,D0

ASL.L	#2,D0

SWAP	D0

ASL.W	#1,D0

ADD.W	D0,FX1		* FX = THRUST*AX + QW*AZ

MOVE.W	BZ(A0),D0

MULS	D1,D0

ASL.L	#2,D0

SWAP	D0

ASL.W	#1,D0

ADD.W	D0,FY1		* FY = THRUST*BX + QW*BZ

MOVE.W	CZ(A0),D0

MULS	D1,D0

ASL.L	#2,D0

SWAP	D0

ASL.W	#1,D0

SUB.W	GRAV,D0

ADD.W	D0,FZ1		* FZ = THRUST*CX + QW*CZ - GRAVITY

FORCE:

MOVE.W	FSHIFT,D6	* Scale the force

CLR.L	D0

MOVE.W	FX1,D0

SWAP	D0

SUB.L	VX1,D0

ASR.L	D6,D0

ADD.L	D0,VX1		* VX = VX + (FX-VX)*K = VX + K*FX - K*VX

CLR.L	D0

MOVE.W	FY1,D0

SWAP	D0

SUB.L	VY1,D0

ASR.L	D6,D0

ADD.L	D0,VY1		* VY = VY + (FY-VY)*K = VY + K*FY - K*VY

CLR.L	D0

MOVE.W	FZ1,D0

SWAP	D0

SUB.L	VZ1,D0

ASR.L	D6,D0

ADD.L	D0,VZ1		* VZ = VZ + (FZ-VZ)*K = VZ + K*FZ - K*VZ

*Update the position

POS:

MOVE.W	PSHIFT,D6

MOVE.L	VX1,D0

ASR.L	D6,D0

ADD.L	D0,XT1			* XT = XT + K*VX

MOVE.L	VY1,D0

ASR.L	D6,D0

ADD.L	D0,YT1			* YT = YT + K*VY

MOVE.L	VZ1,D0

ADDQ.L	#2,D6

ASR.L	D6,D0

ADD.L	D0,ZT1			* ZT = ZT + K*VZ

MOVE.W	XT1,XT(A0)

MOVE.W	YT1,YT(A0)

MOVE.W	ZT1,ZT(A0)

MOVE.W	VX1,VX(A0)		* CHANGE THIS LATER

MOVE.W	VY1,VY(A0)		*      (FOR GUNS)

MOVE.W	VZ1,VZ(A0)

CMP.L	#$400000,ZT1

BGE	POS2

CLR.L	VZ1			* ZERO VZ ON THE GROUND

MOVE.L	#$400000,ZT1		* Eye level

POS2:	NOP

*-----------------------------------------------------------------------------

* Working set is in A0

* Primary Unit Vectors will be derived

DISP	SET	UV2-UV1	  * The working set is always DISP below the working set

YWIND

*	MOVE.W	CY(A0),D0

*	MULS	ASPD,D0

*	ASL.L	#2,D0

*	SWAP	D0

*--------

MOVE.W	YP,D0		*

*--------

MULS	#$800,D0

SWAP	D0

*YAW TIC

* D0 CONTAINS DELTA ANGLE FOR YAW, +/- 79 MAX

* A0=Ship 2 primary

YW3	SUB.L	#DISP,A0

JSR	T_YAW

RTS

*-----------------------------------------------------------------------------

PAGE

* Calculate velocity components in the Ship's reference

* A0 points to Unit Vectors

* X component in XP, Y component in YP, Z component in ZP

VCOM	MOVE.W	VX1,D1

MULS	AX(A0),D1		*D1 = VX*AX

MOVE.W	VY1,D0

MULS	BX(A0),D0

ADD.L	D0,D1			*D1 = VX*AX + VY*BX

MOVE.W	VZ1,D0

MULS	CX(A0),D0

ADD.L	D0,D1			*D1 = VX*AX + VY*BX + VZ*CX

ASL.L	#2,D1

SWAP	D1			*D1 = VX*AX + VY*BX + VZ*CX

NEG.W	D1

MOVE.W	D1,XP			*XP = VXCOM = VX*AX + VY*BX + VZ*CX

*------------

MOVE.W	VX1,D1

MULS	AY(A0),D1		*D1 = VX*AY

MOVE.W	VY1,D0

MULS	BY(A0),D0

ADD.L	D0,D1			*D1 = VX*AY + VY*BY

MOVE.W	VZ1,D0

MULS	CY(A0),D0

ADD.L	D0,D1			*D1 = VX*AY + VY*BY + VZ*CY

ASL.L	#2,D1

SWAP	D1			*D1 = VX*AY + VY*BY + VZ*CY

NEG.W	D1

MOVE.W	D1,YP			*YP = VYCOM = VX*AY + VY*BY + VZ*CY

*------------

MOVE.W	VX1,D1

MULS	AZ(A0),D1		*D1 = VX*AZ

MOVE.W	VY1,D0

MULS	BZ(A0),D0

ADD.L	D0,D1			*D1 = VX*AZ + VY*BZ

MOVE.W	VZ1,D0

MULS	CZ(A0),D0

ADD.L	D0,D1			*D1 = VX*AZ + VY*BY + VZ*CZ

ASL.L	#2,D1

SWAP	D1			*D1 = VX*AZ + VY*BY + VZ*CZ

NEG.W	D1

MOVE.W	D1,ZP			*ZP = VZCOM = VX*AZ + VY*BY + VZ*CZ

RTS

*-----------------------------------------------------------------------------

PAGE

FLY5:

* KISS Airplane Aerodynamics for Ship 1

* PITCH TIC

* D0 CONTAINS DELTA ANGLE FOR PITCH, +/- 79 MAX

MOVEA.W	#UV1,A0			* A0=Ship 1 primary

MOVE.B	RPOT0,D0

SUB.B	#$80,D0			* 80 max

EXT.W	D0

MULU	#$2C,D0			* 1600 max

MOVE.W	CX(A0),D1		* 4000 max

ASR.W	#1,D1			* 2000 max

SUB.W	D1,D0			* error amplitude, 3600 max

ASR.W	#8,D0			* 		   36 max

JSR	T_PTCH

* ROLL TIC

* D0 CONTAINS DELTA ANGLE FOR ROLL, +/- 79 MAX

MOVEA.W	#UV1,A0			*A0=Ship 1 primary

MOVE.B	RPOT1,D0

SUB.B	#$80,D0			* 80 max

EXT.W	D0

MULU	#$2C,D0			* 1600 max

MOVE.W	CY(A0),D1		* 4000 max

ASR.W	#1,D1			* 2000 max

ADD.W	D1,D0			* error amplitude, 3600 max

ASR.W	#8,D0			* 		   36 max

JSR	T_ROLL

* Calculate Force due to Thrust. Distribute thrust along (AX,BX,CX)

MOVEA.W	#UV2,A0		*UV2 is the Working Set

* Thrust

MOVE.W	THRUST(A0),D1

MULS	TSCALE,D1

ASL.L	#2,D1

SWAP	D1		*D1 = scaled thrust

MOVE.W	AX(A0),D0

MULS	D1,D0

ASL.L	#2,D0		* AX * THRUST, MAX = 4000 0000

SWAP	D0

MOVE.W	D0,FX1		* FX = THRUST*AX, MAX = 4000

MOVE.W	BX(A0),D0

MULS	D1,D0

ASL.L	#2,D0		* BX * THRUST, MAX = 4000 0000

SWAP	D0

MOVE.W	D0,FY1		* FY = THRUST*BX, MAX=4000

MOVE.W	CX(A0),D0

MULS	D1,D0

ASL.L	#2,D0		* CX * THRUST, MAX = 1000 0000

SWAP	D0

MOVE.W	D0,FZ1		* FZ = THRUST*CX, MAX=4000

*WING

BSR	VCOM

MOVE.W	XP,D0		* USE VXCOM FOR AIRSPEED

TST.W	D0

BPL	WNG2A

NEG.W	D0

WNG2A	SUB.W	ASPD,D0

ASR.W	#3,D0

ADD.W	D0,ASPD		* ASPD = (NEW-OLD)/8 + OLD

MOVE.W	ZP,D1		* D1= - VZCOMP = QW

MOVE.W	AZ(A0),D0

MULS	D1,D0

ASL.L	#2,D0

SWAP	D0

ASL.W	#1,D0

ADD.W	D0,FX1		* FX = THRUST*AX + QW*AZ

MOVE.W	BZ(A0),D0

MULS	D1,D0

ASL.L	#2,D0

SWAP	D0

ASL.W	#1,D0

ADD.W	D0,FY1		* FY = THRUST*BX + QW*BZ

MOVE.W	CZ(A0),D0

MULS	D1,D0

ASL.L	#2,D0

SWAP	D0

ASL.W	#1,D0

SUB.W	GRAV,D0

ADD.W	D0,FZ1		* FZ = THRUST*CX + QW*CZ - GRAVITY

*FORCE:

MOVE.W	FSHIFT,D6

CLR.L	D0

MOVE.W	FX1,D0

SWAP	D0

SUB.L	VX1,D0

ASR.L	D6,D0

ADD.L	D0,VX1		* VX = VX + (FX-VX)*K = VX + K*FX - K*VX

CLR.L	D0

MOVE.W	FY1,D0

SWAP	D0

SUB.L	VY1,D0

ASR.L	D6,D0

ADD.L	D0,VY1		* VY = VY + (FY-VY)*K = VY + K*FY - K*VY

CLR.L	D0

MOVE.W	FZ1,D0

SWAP	D0

SUB.L	VZ1,D0

ASR.L	D6,D0

ADD.L	D0,VZ1		* VZ = VZ + (FZ-VZ)*K = VZ + K*FZ - K*VZ

*Update the position

*POS

MOVE.W	PSHIFT,D6

MOVE.L	VX1,D0

ASR.L	D6,D0

ADD.L	D0,XT1			* XT = XT + K*VX

MOVE.L	VY1,D0

ASR.L	D6,D0

ADD.L	D0,YT1			* YT = YT + K*VY

MOVE.L	VZ1,D0

ADDQ.L	#2,D6

ASR.L	D6,D0

ADD.L	D0,ZT1			* ZT = ZT + K*VZ

MOVE.W	XT1,XT(A0)

MOVE.W	YT1,YT(A0)

MOVE.W	ZT1,ZT(A0)

MOVE.W	VX1,VX(A0)		* CHANGE THIS LATER

MOVE.W	VY1,VY(A0)		*      (FOR GUNS)

MOVE.W	VZ1,VZ(A0)

CMP.L	#$400000,ZT1

BGE	POS2A

CLR.L	VZ1			* ZERO VZ ON THE GROUND

MOVE.L	#$400000,ZT1		* Eye level

POS2A:

*-----------------------------------------------------------------------------

* Working set is in A0

* Primary Unit Vectors will be derived

DISP	SET	UV2-UV1	  * The working set is always DISP below the working set

*YWIND

*	MOVE.W	CY(A0),D0

*	MULS	ASPD,D0

*	ASL.L	#2,D0

*	SWAP	D0

*--------

MOVE.W	UV2+CY,D0		*

*--------

MULS	#$30,D0

SWAP	D0

*YAW TIC

* D0 CONTAINS DELTA ANGLE FOR YAW, +/- 79 MAX

* A0=Ship 2 primary

*YW3

SUB.L	#DISP,A0

JSR	T_YAW

RTS

*===============================================================================