* TomCat Hardware Memory Map, PC Board

* 68010 Main Processor

*------------------------------------------------------------------------------

Program EPROM ( 16 x 27256 max):

ROM 0	00 000 - 0F FFF	 	R

ROM 1 	10 000 - 1F FFF	 	R

ROM 2	20 000 - 2F FFF	 	R

ROM 3	30 000 - 3F FFF	 	R

ROM 4	40 000 - 4F FFF	 	R

ROM 5	50 000 - 5F FFF	 	R

ROM 6	60 000 - 6F FFF	 	R

ROM 7	70 000 - 7F FFF	 	R

*------------------------------------------------------------------------------

LS Bus:

LNKRD	400 000	 R	Read Game Link Data

ADCREAD	402 000	 R	Read A/D Converter output

INPUTS	404 000	 R	Switch Inputs

D15	/L THUMB		D7	/DSW 7

D14	/R THUMB		D6	/DSW 6

D13	/L FIRE			D5	/DSW 5

D12	/R FIRE			D4	/DSW 4

D11	/SELF TEST		D3	/DSW 3

D10	/DIAGN.			D2 	/DSW 2

D9	/SPARE			D1 	/DSW 1

D8	VGHALT			D0	/DSW 0

PRNG	406 000	 R	Read Pseudo-Random-Number Generator (Word)

IN1	408 000	 R	Switch Inputs

D15	LNKFLAG		(Game Link)

D14	PC3		   "    "

D13	PC2		   "    "

D12	PC0		   "    "

D11	MAINFLAG	(Sound System)

D10	SOUNDFLAG	   "    "

D9	/IDLE*		(TMS320 System)

D8

320BIO	40A 000	 R	Start TMS-320 Program (Address Strobe)

MAINRD	40C 000	 R	Read Sound Interface Data

PRNGCLR 40E 000	 R	Reset PRNG (Address Strobe)

LNKWR	400 000	 W	Write Game Link Data

ADCON	402 000	 W	A/D Control; Set up address, then D3 high, then D3 low

D0	ADDA

D1	ADDB

D2	ADDC

D3	ALE/START

VGGO	404 000	 W	Start AVG	(Address Strobe)

VGRST	406 000	 W	Reset AVG	(Address Strobe)

WDCLR	408 000	 W	Clear Watch Dog	(Address Strobe)

IRQCLR	40A 000	 W	Clear IRQ	(Address Strobe)

MAINWR	40C 000	 W	Write to Sound Main Latch

LEDS	40E 000	 W	Latched outputs	(Address Strobe)

On		  Off

-------		-------

LED 1		40E 000	 	40E 010

LED 2		40E 002		40E 012

320RES		40E 004		40E 014		Reset TMS-320 System

SNDRES		40E 006		40E 016		Reset Sound System

LNKMODE		40E 008		40E 018		Game Link

ERR		40E 00A		40E 01A		  "    "

ACK		40E 00C		40E 01C		  "    "

TXBUFF		40E 00E		40E 01E		  "    "

*------------------------------------------------------------------------------

VMEM	800 000 - 803 FFF	Vector Generator RAM, 16K Bytes

*------------------------------------------------------------------------------

320RAM	0FF A000 - 0FF DFFF	TMS-320 RAM, 16K Bytes

*------------------------------------------------------------------------------

RAM 0	0FF C000 - 0FF CFFF 	4K Bytes

RAM 1	0FF D000 - 0FF DFFF 	4K Bytes Regular or Mostek Zero-Power

*------------------------------------------------------------------------------

* TMS-320 Memory in 68010 space, not including offset:

*

*	     0 - 7	Idle Loop		[4 words]

*	     8 - 0B 	Branch Instruction   	[2 words]

*	    0C - 0FF 	Direct Ram		[122 words]

*	   100 - 13FF 	Program			[2432 words]

*	  1400 - 17FF 	Output Buffer		[512 words]

*	  1800 - 1FFF 	Object Pictures 	[1024 words]

*

*------------------------------------------------------------------------------

* TomCat Hardware Memory Map, PC Board

6502 Sound Processor

0000 - 1FFF	RAM (8K bytes)

2000		Yamaha

3000		XLOAD 0		Write the Sequential ROM counter Low Byte

3020		XLOAD 1		  "    "       "      "     "    High  "

3040		SOUNDWR		Write to Sound Interface Latch (read by Main)

3060		unused

3080		XREAD		Read the Sequential ROM (64K bytes) and

increment the counter

30A0		unused

30C0		SOUNDREAD	Read the Sound Interface Latch (written by Main)

30E0		COINRD		Inputs:

D7 = Coin L

D6 = Coin R

D5 = SOUNDFLAG

4000		unused

================================================================================

5000	(PIA)

5000 - 507F	RAM	(128 BYTES)

-----------

5080	PAD

PA0 = /WS	OUTPUT	(TMS-5220 WRITE STROBE)

PA1 = /RS	OUTPUT	(TMS-5220 READ STROBE)

PA2 = /READY	INPUT	(TMS-5220 READY FLAG)

PA3 = FSEL	OUTPUT	Select TMS5220 clock;

0 = 325 KHz (8 KHz sampling)

1 = 398 KHz (10 KHz sampling)

PA4 = /CC1	OUTPUT	Coin Counter 1

PA5 = /CC2	OUTPUT	Coin Counter 2

PA6 = /MUSRES 	OUTPUT  (Reset the Yamaha)

PA7 = MAINFLAG	INPUT

5081	PADD	(=7B)	PAD Direction

-----------

5082	PBD

PB0 - PB7	OUTPUT	Speech Data

5083	PBDD	(=FF)	PBD Direction

-----------

5084	RT  	;READ TIMER, RESET AND DISABLE TIMER IRQ

5085	RIF 	;READ INTERRUPT FLAG - D7=TIMER FLAG, D6=PA7 FLAG

5086	EINED	;ENABLE INTERRUPTS WITH NEGATIVE EDGE DETECT

5087	EIPED	;ENABLE INTERRUPTS WITH POSITIVE EDGE DETECT

================================================================================

6000		POKEY 0

7000		POKEY 1

8000 - FFFF	Sound Program ROM (32K bytes)